Goods Inwards

Check for the following, if any errors occur, complete ‘Non Conformity Form 001’ and report to Factory Manager or Director.

- Office Administrator checks Purchase Order against Supplier Delivery Note.

- Check Supplier Delivery Note for the following:

  - Quantity.

  - Correct Standards.

  - Delivery Date.

  - As per Purchase Orders.

  - Batch Numbers.

  - Supplier Purchase Order Number.

- Director and or Foreman informed of any discrepancies.

- Report Delivery to Line Manager

Forms – All Completed Forms returned to Factory Manager or Director

- Purchase Order		Form 012

- Non Conformity 		Form 001

- Supplier Delivery Note	Form 022

Inspection

- Glass

  - Dimensions.

  - Qty.

  - Visual appearance – Scratches, Sealant, Georgian Bar Alignment.

- Profile

  - Batch Numbers – Visual appearance of packaging and quantity.

- Hardware

  - Qtys

  - Standards,

  - Description

Check Delivery Book at 4pm each day and Report any outstanding

Purchase Orders to author of Order immediately.

Profile Saw

Check for the following, if any errors occur, complete ‘Non Conformity Form 001’ and report to Supervisor.

- Ensure correct Profile/saw support block is used where applicable.

- Visually inspect Profile before Cutting for Damage etc.

- Check Material codes against Cutting List.

- Check profile matches profile chart.

- Check cutting list for:

  - Length.

  - Angle.

  - Qty.

- Tolerances of +/- 0.5mm on length.

- Check saw blocks are correct for profile being cut

- Check angle accuracy as per ‘Smart Systems Limited’ wall chart.

- After first cut of the New Profile, measure with tape & record

- Each Cut piece placed in Job Bin.

- Sign Cutting List after every job completion.

- Completed job moved onto next Work Station as per Cutting List in bins.

Forms – Attach to Board

- Cutting List			Form 020

- Non Conformity Forms	Form 001 - When completed, pass to Factory Mgr.

- Profile First off record	Form 014 - Pass to Factory Mgr at End of Week.

Wall charts

- To be determined by Manufacturer and ‘Smart Systems Limited’.

Fabrication Manual

- With Bench 1

Inspection

Ensure all saw support blocks are correct.

Profile Condition.

Blade Condition & renewal/change date.

Ensure Cutting Fluid levels are correct in container and check Stock level of Cutting Fluid.

Check material for burrs.

Ensure Saw Bed is clean before cutting NEW bar length.

Milling & Routing

- Ensure Paperwork is complete.

- Check profile code matches wall chart.

- Check profile for burrs and general condition of profile.

- Profile put into m/c & mill as per Smart Systems Limited Fabrication Manual.

- Visually check for burrs and general running of m/c.

- Sign Cutting List upon Job Completion.

- Return completed profile into bins and passed onto next workstation as per Cutting List.

Forms – attach to Board

- Cutting List			Form 020 - When completed, attach to Board 2.

- Non Conformity Forms	Form 001 - When completed, pass to Factory Mgr.

- Mill First off record		Form 015 - Pass to Factory Manager at End of Week.

Wall charts

- To be determined by Manufacturer and ‘Smart Systems Limited’.

Fabrication Manual

- With Bench 1

Inspection

Ensure machine is clean and clear before insertion of Profile

Condition of received profile.

Condition of Blades, Cleanliness and general running of m/c.

When a blade is changed, check for:

  - Transom to Frame.

  - Transom to Transom.

  - Burrs.

Ensure Machine is clean at the start of each shift

Drainage

- Check paperwork is complete.

- Check qty’s and codes are as per cutting list.

- Check condition of profile.

- Refer to Drainage Booklet.

- Drill slot hole as per ‘Smart Systems Limited’ instructions.

- Check for burrs.

- Sign Cutting List.

- Completed profile returned to bin and passed onto next workstation.

Forms – Attach to Board

- Cutting List			Form 020 - When completed, attach to Board 2.

- Non Conformity Forms	Form 001 - When completed, pass to Factory Mgr.

- Mill First off record		Form 014 - Pass to Factory Mgr at End of Week

Wall charts

- To be determined by Manufacturer and ‘Smart Systems Limited’.

Fabrication Manual

- With Bench 1

Inspection

Check condition of Blade and m/c in general.

Check condition of profile.

Ensure machine is clear and clean before inserting profile.

Ensure Machine is clean at the start of each shift

Conventional Window – Pre Assembly

- Check paperwork is complete.

- Check qty’s and codes are as per cutting list.

- Check condition of profile, Drainage & Routing slots for burrs.

- Obtain Gasket as per cutting List.

- Check dates on Gasket Buckets.

- Check Gasket against ‘Gasket Chart’.

- Gasket & Bubble Seal, then inserted into Extrusion as per ‘Smart Systems Limited’

Fabrication Manual.

- Cut and mitred to section.

- Ensure Gasket and seal are flush, cut with a sharp blade.

- Sign Cutting List.

- Completed profile returned to bin and passed onto next workstation.

Forms – Attach to Board

- Cutting List			      Form 020.

- Non Conformity Forms	      Form 001 - When completed, pass to Factory Manager.

- Pre Assembly First off record   Form 017 - Pass to Factory Manager at End of Week

Wall charts

- To be determined by Manufacturer and ‘Smart Systems Limited’.

Fabrication Manual

- With Bench 1

Inspection

Condition of Profile.

Gasket Stock Level.

Crimper Bench

- Check paperwork is complete.

- Check qty’s and codes are as per cutting list.

- Check Cleats as per cutting List against Cleat Chart.

- Ensure Smart Systems Limited Crimper Tooling as per records are in place and accessible at all times.

- Set up Crimping Machine as per the Wall Chart.

- Check sample is OK and adjust Crimping machine if necessary.

- Insert Corner Cleat and Corner Chevrons.

- Silicone Cut Faces of mitre joint and wipe off excess Silicone

- Seal Mitre faces.

- Position Mitre and Crimp.

- Check strength of mitre joint (Should NOT Move), Chevrons should not rattle!! Crimp should be GAP Free and flush internally and externally.

- Sign Cutting List.

- Assembly then passed onto Next Work Station as per Cutting List.

Forms – Attach to Board

- Cutting List				 	Form 020.

- Non Conformity Forms		 	Form 001 -  When completed, pass to

Factory Manager.

- Crimped Assembly First off record     	Form 015 -  Pass to Factory Manager at 								Week End.

Wall charts

- To be determined by Manufacturer and ‘Smart Systems Limited’.

Fabrication Manual

- With Bench 1

Inspect

- Condition of Profile, Gasket and Bubble Seal.

- Cleat Stock Levels.

- Chevron Stock Levels.

- Mitre Lines.

Conventional Window – Part Build

- Check paperwork is complete.

- Check Profile codes for ‘Transoms’ and ‘Mullions’.

(Check against Profile Chart on the wall).

- Use Drill Jig as per ‘Tooling Chart’.

- Align and Drill accordingly as per Smart Systems Limited Fabrication manual.

- Insert Bubble Seal.

- Locate Transom/Mullion within Frame member.  Ensure both inner and outer faces are flush with section when located and fix using screws as per ‘Assembly Section’ in Smart Systems Limited fabrication manual

- Sign Cutting List.

- Assembly then passed onto Next Work Station as per Cutting List.

Forms – Attach to Board

- Cutting List				     Form 020

- Non Conformity Forms		     Form 001 - When completed, pass to

Factory Manager.

- Window Pre Build Record		     Form 018 -  Pass to Factory Manager

at End of Week.

Wall charts

- To be determined by Manufacturer and ‘Smart Systems Limited’.

Fabrication Manual

- With Bench 1

Inspect

- Cutting List.

- Drainage Holes.

- Routing Slots.

- Burrs.

- Stock Levels for Screws etc

Conventional Window – Final Assembly

- Check paperwork is complete.

- Visually Inspect Frame For:

        - Damage.

        - Poor Quality Assembly.

        - Gaps.

- Check Locking Mechanisms etc as per Cutting List.

- Assemble Locking Mechanisms into Vent Frame as per manual.

- Assemble Friction Stays onto Vent Sections using.

- Fit Handle as per manual

- ‘Keep’ locations as per manual.

- Sign Cutting List.

- Assembly then passed onto Next Work Station as per Cutting List.

Forms – Attach to Board

- Cutting List				     Form 020.

- Non Conformity Forms		     Form 001 -  When completed, pass to

Factory Manager.

- Window Final assembly Record	     Form 019 -  Pass to Factory Manager

at End of Week

Wall charts

- To be determined by Manufacturer and ‘Smart Systems Limited’.

Fabrication Manual

- With Bench 1

Inspection

Monitor Stock Levels of:

  - Locks.

  - Handles.

  - Screws.

Window Operation Checks

Check Sash Alignment through Mitres.  Sash operation should be clear with no fouling

Conventional Window – Beading

- Check paperwork is complete.

- Visually Inspect Assembly For:

  - General Damage

  - Gaps in Finished Beads

- Beading completed using Measuring Stick.

- Beads as Cut as per the following:

  - Bottom - Both Ends Square.

  - Sides   - Scribe Bottom, Square Top.

  - Top      - Both Ends Scribe.

- Fit Beads and then remove.

- Obtain ‘Captive Rubber Gasket’.

- Insert ‘Captive Rubber Gasket’ as per Cutting List.

- Reinsert Bead and secure for Transportation.

- Attach Glazing Wedge to Window for Dispatch.

- Sign Cutting List.

- Assembly then passed onto Next Work Station as per Cutting List.

Forms – Attach to Board

- Cutting List				     Form 020 - When Completed, pass to

Final Inspection.

- Non Conformity Forms		     Form 001 - When completed, pass to

Factory Manager.

Wall charts

- To be determined by Manufacturer and ‘Smart Systems Limited’.

Fabrication Manual

- With Bench 1

Inspection

Check Profile Condition.

Monitor Stock Levels of Beads.

Check Beads for Burrs.

Check Bead Saw Blade and date for Change.

Ensure Internal Glazing wedge is attached to window for dispatch.

Conventional Window – Full Inspection

- Check paperwork is complete.

- Check quantity of Windows and Doors

- Visually Inspect Assembly For:

  - Damage.

  - Gaps.

  - Finnish.

- Check mitre lines are true.

- Check Full Operation of Window.

- Full Dimensional Check. +/- 1.5mm.

- Check Glass, as per Cutting List on Board.

- Check Handle type and colour.

- If the Window passes, stick on ‘Green Sticker’.

- If product passes, apply Label.

- If the Window Fails, stick on ‘Red Sticker’, raise Non Conformity,

inform Factory Manager and place in Quarantine area.

- Sign Cutting List.

- Assembly then passed onto Despatch Station as per Cutting List.

Forms – Attach to Board

- Cutting List				     Form 020 - When completed, pass onto 									Despatch.

- Non Conformity Forms		     Form 001 - When completed, pass to

Factory Manager.

Charts

- To be determined by Manufacturer and ‘Smart Systems Limited’.

Inspection

As Above

Window – Rebate Bead

- Check paperwork is complete.

- Visually Inspect Assembly For:

  - General Damage

  - Gaps in Finished Beads

- Beading completed using Measuring Stick

- Beads as Cut as per the following:

  - Top and Bottom - Both Ends Square.

  - Sides   - Scribe Both Ends

- Fit Beads and then remove.

- Obtain ‘Captive Rubber Bubble Seal’, check Batch Number and Date.

- Insert ‘Captive Rubber Bubble Seal’ as per Cutting List.

- Reinsert Bead and secure with Rebate Wedge.

- Sign Cutting List.

- Assembly then passed onto Next Work Station as per Cutting List.

Forms – Attach to Board

- Cutting List				     Form 020

- Non Conformity Forms		     Form 001 -  When completed, pass to

Factory Manager.

Wall charts

- To be determined by Manufacturer and ‘Smart Systems Limited’.

Fabrication Manual

- With Bench 1

Inspection

Check Profile Condition.

Check Beads for Burrs.

Check Bead Saw Blade and date for Change.

Monitor Stock Levels for ‘Rebate Wedge’.

Entrance Door - Preparation

- Check paperwork is complete.

- Drill Pilot Holes as per ‘Smart Systems Limited’ Tooling Chart/Fabrication Manual’ using appropriate drill jig.

- Tolerances +/- 1mm

- Check for Burrs.

- Sign ‘Cutting List.

- Assembly then passed onto Next Work Station as per Cutting List.

Forms – Attach to Board

- Cutting List				     Form 020

- Non Conformity Forms		     Form 001  When completed, pass to

Factory Manager.

Wall charts

- To be determined by Manufacturer and ‘Smart Systems Limited’.

Fabrication Manual

- With Bench 1

Inspect

Profile Condition

Entrance Door – Partial Assembly

- Check paperwork is complete.

- Obtain ‘Captive Gasket’ as per cutting List & Insert.

- Assemble corners of door leaf ensuring ALL components detailed within

‘Smart Systems Limited’ Fabrication Manual.

- Glaze the Door at the Letter ‘E’ stage of assembly as per ‘Smart Systems Limited’ Fabrication Manual.  Ensure the Glass Packers are in place at this point.

Remaining Vertical Section then assembled.

- Sign ‘Cutting List.

- Assembly then passed onto Next Work Station as per Cutting List.

Forms – Attach to Board

- Cutting List				     Form 020

- Non Conformity Forms		     Form 001 -  When completed, pass to

FactoryManager.

Wall charts

- To be determined by Manufacturer and ‘Smart Systems Limited’.

Fabrication Manual

- With Bench 1

Inspection

- Monitor Captive Gasket Stock Level.

Entrance Door – Final Assembly

- Check Paperwork is complete.

- Assemble Hinge Back Plates into Frame as per ‘Smart Systems Limited’ Fabrication

Manual.

- Assemble Lock Keep Back Plates into Frame as per manual.

- Insert Woolpile Seal into Sections.

- Assemble Outer Frame Components and using Corner Cleats and Cams as shown

in the Smart Systems Limited’ Fabrication Manual, applying a silicone seal prior to

final tightening.

- Fix Hinge to Outer Frame using the back plate and Machine Screws provided.

- Adjust hinges to final fix position and secure

- Lift door leaf onto hinges and adjust accordingly.

- Fix coverstrip throughout where necessary.

- When Complete, pass onto Final Inspection.

Forms – Attach to Board

- Cutting List				     Form 020 – When complete, pass onto

Final Inspection.

- Non Conformity Forms		     Form 001 - When completed, pass to

Factory Manager.

Wall charts

- To be determined by Manufacturer and ‘Smart Systems Limited’.

Fabrication Manual

- With Bench 1